Firmware Development and Implementation

Week 2 Task Report – Embedded Systems Internship

Module: Fault Indicator LED Controller (Timer Interrupt + UART)

Submitted by: Nitish Krishna M H

B.E. Electronics and Communication Engineering

Sri Ramakrishna Engineering College, Coimbatore

# 1. Introduction

Building on the Week 1 requirements analysis for the Smart Asset Safety and Data Logging System, this task moves from planning into hands-on firmware development. Rather than implementing the entire system at once, a single, self-contained module was selected that still exercises the two core mechanisms the full project depends on: a hardware timer interrupt and UART communication.

The module implemented here is a Fault Indicator LED Controller. A status LED blinks at a rate that reflects the asset's current condition — slow for normal operation, faster as the condition worsens — while a short status message is transmitted over UART whenever that condition changes. This mirrors, at a small scale, exactly how the larger system is meant to behave: sense a value, classify it, react through an output, and report the change to a remote observer.

As physical hardware was not available for this task, the firmware was written against the standard STM32 HAL API and exercised through simulation, as described in Section 5.

# 2. Firmware Architecture

The firmware is organised around one interrupt source and one polling loop, kept deliberately separate so neither can block the other:

- A 100 ms TIM2 interrupt drives all LED timing. The ISR only counts ticks and toggles a pin — it never touches the sensor or UART, so it stays short and predictable, the same principle used for the Fault Task in the full project's FreeRTOS design.

- The main loop polls the (simulated) ADC sensor every 200 ms, classifies the reading into a fault level, and — only when that level actually changes — updates the blink rate the ISR uses and sends one UART status line.

- A single volatile state variable (blinkTickTarget) is the only data shared between the ISR and the main loop, which keeps the interrupt-safety concerns minimal and easy to reason about.

The key functions and their roles are summarised below.

| Function | Role |
| --- | --- |
| GPIO_LED_Init() | Configures the status LED pin as a push-pull digital output. |
| TIM2_Init() | Configures TIM2 for a 100 ms periodic update interrupt — the blink 'heartbeat'. |
| UART2_Init() | Configures USART2 at 9600 baud for transmitting status messages. |
| ADC1_Init() | Configures ADC1 channel 0, which stands in for a real sensor input during simulation. |
| Read_Simulated_Sensor() | Runs a single ADC conversion and returns the raw 0-4095 reading. |
| Evaluate_Fault_Level() | Maps the raw reading onto NORMAL / WARNING / CRITICAL using threshold constants. |
| Send_UART_Status() | Builds and transmits a short status string over UART. |
| TIM2_IRQHandler() | ISR: counts 100 ms ticks and toggles the LED once the current blink target is reached. |
| main() | Initializes all peripherals, then polls the sensor and updates state only on a change. |

## 2.1 Fault Level Thresholds

The simulated 12-bit ADC reading (0-4095) is split into three bands: below 1500 is treated as NORMAL, 1500-2999 as WARNING, and 3000 and above as CRITICAL. These bands drive both the LED blink interval (500 ms, 200 ms, and 100 ms respectively) and the point at which a UART status message is sent.

# 3. Firmware Code Listing

The complete, commented source is reproduced below and is also included in the project GitHub repository as fault_led_controller.c.

1 /* ===================================================================================== 2 * File : fault_led_controller.c 3 * Project : Smart Asset Safety & Data Logging System — Week 2 Firmware Task 4 * Author : Nitish Krishna M H 5 * Target : STM32F4xx (Cortex-M4), STM32 HAL-style API 6 * Toolchain : STM32CubeIDE (GCC ARM) — code was simulated using the STM32CubeIDE 7 * peripheral simulation / SWV console, since physical hardware was not 8 * available for this task. 9 * 10 * --------------------------------------------------------------------------------------- 11 * README (also see week2 folder README.md in the GitHub repo for the same content) 12 * --------------------------------------------------------------------------------------- 13 * WHAT THIS FILE DOES 14 * This is a scaled-down firmware module taken from the larger Smart Asset Safety 15 * project. It implements a "Fault Indicator LED Controller": 16 * - A hardware timer (TIM2) fires a periodic interrupt that toggles a status LED. 17 * - The BLINK RATE of the LED changes depending on the current fault level of the 18 * asset (NORMAL / WARNING / CRITICAL), so a person can tell the asset's health 19 * just by looking at the LED. 20 * - Whenever the fault level changes, a short status message is transmitted over 21 * UART (this stands in for the GSM alert link in the full project). 22 * - The fault level itself is derived from a simulated analog sensor reading 23 * (ADC channel), since no physical temperature/current sensor was connected 24 * during this simulated test. 25 * 26 * KEY FUNCTIONS 27 * GPIO_LED_Init() - configures the LED pin as a digital output 28 * TIM2_Init() - configures TIM2 to generate an interrupt every 100 ms 29 * UART2_Init() - configures USART2 at 9600 baud for status messages 30 * ADC1_Init() - configures ADC1 channel 0 to read the simulated sensor 31 * Read_Simulated_Sensor()- returns a 0-4095 ADC-style value representing sensor input 32 * Evaluate_Fault_Level() - converts the sensor reading into NORMAL/WARNING/CRITICAL 33 * Send_UART_Status() - transmits a short status string over UART 34 * TIM2_IRQHandler() - ISR: toggles the LED at a rate set by the current fault level 35 * main() - initializes peripherals and runs the sensor-polling loop 36 * 37 * HOW TO COMPILE / SIMULATE 38 * 1. Create a new STM32F4 project in STM32CubeIDE (any STM32F4 Nucleo board target 39 * works, e.g. NUCLEO-F401RE) and enable CubeMX code generation for GPIO, TIM2, 40 * USART2 and ADC1 (defaults are fine — this file only depends on the standard 41 * HAL calls shown below). 42 * 2. Replace the auto-generated Core/Src/main.c body with this file's contents 43 * (or add it as a separate .c file and call Fault_LED_Controller_Main() from 44 * the generated main()). 45 * 3. Build the project (Project -> Build). No external libraries are required 46 * beyond the STM32F4xx HAL that CubeMX generates. 47 * 4. To simulate without hardware: use STM32CubeIDE's built-in Debug perspective 48 * with the SWV (Serial Wire Viewer) console to watch UART_Status_Log prints, 49 * or load the .elf into a simulator such as Renode / Proteus VSM, which can 50 * emulate the STM32F4 peripherals and show the LED pin toggling and the UART 51 * output on a virtual terminal. 52 * 5. During simulation, the simulated ADC input can be swept from 0 to 4095 to 53 * manually walk the system through NORMAL -> WARNING -> CRITICAL and observe 54 * both the LED blink-rate change and the UART message on the virtual terminal. 55 * 56 * TESTING PROCESS (see full write-up in the accompanying Word report) 57 * The design was exercised in three simulated scenarios by varying the simulated 58 * ADC value: (1) steady low reading -> NORMAL, LED blinks slowly, no UART message 59 * after boot; (2) reading raised into the mid band -> WARNING, blink rate doubles 60 * and one UART status line is sent; (3) reading raised further -> CRITICAL, blink 61 * rate is fastest and another UART status line is sent. Returning the reading to 62 * the NORMAL band correctly reset the blink rate and produced a final UART message, 63 * confirming the state machine only sends a message on a *change* of state rather 64 * than on every loop iteration. 65 * ===================================================================================== 66 */ 67 68 #include "stm32f4xx_hal.h" 69 #include <stdio.h> 70 #include <string.h> 71 72 /* --------------------------------------------------------------------------------- 73 * Pin / peripheral configuration 74 * --------------------------------------------------------------------------------- */ 75 #define LED_GPIO_PORT GPIOA 76 #define LED_GPIO_PIN GPIO_PIN_5 /* on-board LED (e.g. Nucleo LD2) */ 77 78 /* Fault threshold bands for the simulated 12-bit ADC reading (0 - 4095) */ 79 #define THRESHOLD_WARNING 1500 80 #define THRESHOLD_CRITICAL 3000 81 82 /* Blink interval, expressed in number of 100 ms timer ticks */ 83 #define BLINK_TICKS_NORMAL 5 /* toggle every 500 ms -> slow blink */ 84 #define BLINK_TICKS_WARNING 2 /* toggle every 200 ms -> medium blink */ 85 #define BLINK_TICKS_CRITICAL 1 /* toggle every 100 ms -> fast blink */ 86 87 /* --------------------------------------------------------------------------------- 88 * Types 89 * --------------------------------------------------------------------------------- */ 90 typedef enum { 91 FAULT_NORMAL = 0, 92 FAULT_WARNING, 93 FAULT_CRITICAL 94 } FaultLevel_t; 95 96 /* --------------------------------------------------------------------------------- 97 * Peripheral handles (populated by CubeMX-style *_Init functions) 98 * --------------------------------------------------------------------------------- */ 99 TIM_HandleTypeDef htim2; 100 UART_HandleTypeDef huart2; 101 ADC_HandleTypeDef hadc1; 102 103 /* --------------------------------------------------------------------------------- 104 * Module state 105 * (declared volatile because it is written inside an ISR and read in main()) 106 * --------------------------------------------------------------------------------- */ 107 static volatile FaultLevel_t currentFaultLevel = FAULT_NORMAL; 108 static volatile uint8_t blinkTickTarget = BLINK_TICKS_NORMAL; 109 static volatile uint8_t blinkTickCounter = 0; 110 111 /* --------------------------------------------------------------------------------- 112 * Function prototypes 113 * --------------------------------------------------------------------------------- */ 114 void SystemClock_Config(void); 115 static void GPIO_LED_Init(void); 116 static void TIM2_Init(void); 117 static void UART2_Init(void); 118 static void ADC1_Init(void); 119 static uint16_t Read_Simulated_Sensor(void); 120 static FaultLevel_t Evaluate_Fault_Level(uint16_t sensorValue); 121 static void Send_UART_Status(FaultLevel_t level); 122 123 /* ===================================================================================== 124 * GPIO_LED_Init 125 * Configures the status LED pin as a push-pull digital output. 126 * ===================================================================================== */ 127 static void GPIO_LED_Init(void) 128 { 129 GPIO_InitTypeDef GPIO_InitStruct = {0}; 130 131 __HAL_RCC_GPIOA_CLK_ENABLE(); 132 133 GPIO_InitStruct.Pin = LED_GPIO_PIN; 134 GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP; 135 GPIO_InitStruct.Pull = GPIO_NOPULL; 136 GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW; 137 HAL_GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct); 138 139 HAL_GPIO_WritePin(LED_GPIO_PORT, LED_GPIO_PIN, GPIO_PIN_RESET); 140 } 141 142 /* ===================================================================================== 143 * TIM2_Init 144 * Configures TIM2 to raise an update interrupt every 100 ms. This tick is the 145 * "heartbeat" the ISR uses to decide when to toggle the LED, based on the current 146 * blinkTickTarget set by the fault level. 147 * ===================================================================================== */ 148 static void TIM2_Init(void) 149 { 150 __HAL_RCC_TIM2_CLK_ENABLE(); 151 152 /* Assuming a 16 MHz APB1 timer clock: 153 * Prescaler divides down to 10 kHz (16,000,000 / 1600 = 10,000 Hz) 154 * Period of 1000 ticks at 10 kHz -> 100 ms interrupt interval */ 155 htim2.Instance = TIM2; 156 htim2.Init.Prescaler = 1600 - 1; 157 htim2.Init.CounterMode = TIM_COUNTERMODE_UP; 158 htim2.Init.Period = 1000 - 1; 159 htim2.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1; 160 HAL_TIM_Base_Init(&htim2); 161 162 HAL_NVIC_SetPriority(TIM2_IRQn, 1, 0); 163 HAL_NVIC_EnableIRQ(TIM2_IRQn); 164 165 HAL_TIM_Base_Start_IT(&htim2); 166 } 167 168 /* ===================================================================================== 169 * UART2_Init 170 * Configures USART2 for 9600-baud, 8N1 status message transmission. 171 * ===================================================================================== */ 172 static void UART2_Init(void) 173 { 174 huart2.Instance = USART2; 175 huart2.Init.BaudRate = 9600; 176 huart2.Init.WordLength = UART_WORDLENGTH_8B; 177 huart2.Init.StopBits = UART_STOPBITS_1; 178 huart2.Init.Parity = UART_PARITY_NONE; 179 huart2.Init.Mode = UART_MODE_TX_RX; 180 huart2.Init.HwFlowCtl = UART_HWCONTROL_NONE; 181 huart2.Init.OverSampling = UART_OVERSAMPLING_16; 182 HAL_UART_Init(&huart2); 183 } 184 185 /* ===================================================================================== 186 * ADC1_Init 187 * Configures ADC1 channel 0 in single-conversion mode. This channel stands in for a 188 * real temperature/current sensor input; during simulation its value is stepped 189 * manually (or via a simulator's virtual potentiometer) to exercise all fault bands. 190 * ===================================================================================== */ 191 static void ADC1_Init(void) 192 { 193 ADC_ChannelConfTypeDef sConfig = {0}; 194 195 __HAL_RCC_ADC1_CLK_ENABLE(); 196 197 hadc1.Instance = ADC1; 198 hadc1.Init.Resolution = ADC_RESOLUTION_12B; 199 hadc1.Init.ScanConvMode = DISABLE; 200 hadc1.Init.ContinuousConvMode = DISABLE; 201 hadc1.Init.DiscontinuousConvMode = DISABLE; 202 hadc1.Init.DataAlign = ADC_DATAALIGN_RIGHT; 203 hadc1.Init.NbrOfConversion = 1; 204 HAL_ADC_Init(&hadc1); 205 206 sConfig.Channel = ADC_CHANNEL_0; 207 sConfig.Rank = 1; 208 sConfig.SamplingTime = ADC_SAMPLETIME_15CYCLES; 209 HAL_ADC_ConfigChannel(&hadc1, &sConfig); 210 } 211 212 /* ===================================================================================== 213 * Read_Simulated_Sensor 214 * Performs a single ADC conversion and returns the 12-bit result (0-4095). 215 * In simulation, this value is driven by the simulator's virtual analog input. 216 * ===================================================================================== */ 217 static uint16_t Read_Simulated_Sensor(void) 218 { 219 uint16_t value = 0; 220 221 HAL_ADC_Start(&hadc1); 222 if (HAL_ADC_PollForConversion(&hadc1, 10) == HAL_OK) 223 { 224 value = (uint16_t)HAL_ADC_GetValue(&hadc1); 225 } 226 HAL_ADC_Stop(&hadc1); 227 228 return value; 229 } 230 231 /* ===================================================================================== 232 * Evaluate_Fault_Level 233 * Maps a raw sensor reading onto one of the three fault levels using the threshold 234 * constants defined above. 235 * ===================================================================================== */ 236 static FaultLevel_t Evaluate_Fault_Level(uint16_t sensorValue) 237 { 238 if (sensorValue >= THRESHOLD_CRITICAL) 239 { 240 return FAULT_CRITICAL; 241 } 242 else if (sensorValue >= THRESHOLD_WARNING) 243 { 244 return FAULT_WARNING; 245 } 246 else 247 { 248 return FAULT_NORMAL; 249 } 250 } 251 252 /* ===================================================================================== 253 * Send_UART_Status 254 * Transmits a short, human-readable status line describing the new fault level. 255 * ===================================================================================== */ 256 static void Send_UART_Status(FaultLevel_t level) 257 { 258 char message[64]; 259 const char *levelText; 260 261 switch (level) 262 { 263 case FAULT_WARNING: levelText = "WARNING"; break; 264 case FAULT_CRITICAL: levelText = "CRITICAL"; break; 265 default: levelText = "NORMAL"; break; 266 } 267 268 snprintf(message, sizeof(message), "STATUS: Asset condition -> %s\r\n", levelText); 269 HAL_UART_Transmit(&huart2, (uint8_t *)message, (uint16_t)strlen(message), 100); 270 } 271 272 /* ===================================================================================== 273 * TIM2_IRQHandler 274 * Fires every 100 ms. Counts ticks until it reaches the current blinkTickTarget 275 * (which depends on the fault level), then toggles the LED and resets the counter. 276 * Keeping the ISR this short ensures it never blocks other time-critical work, 277 * consistent with the FreeRTOS task-priority design used in the full project. 278 * ===================================================================================== */ 279 void TIM2_IRQHandler(void) 280 { 281 if (__HAL_TIM_GET_FLAG(&htim2, TIM_FLAG_UPDATE) != RESET) 282 { 283 __HAL_TIM_CLEAR_FLAG(&htim2, TIM_FLAG_UPDATE); 284 285 blinkTickCounter++; 286 if (blinkTickCounter >= blinkTickTarget) 287 { 288 HAL_GPIO_TogglePin(LED_GPIO_PORT, LED_GPIO_PIN); 289 blinkTickCounter = 0; 290 } 291 } 292 } 293 294 /* ===================================================================================== 295 * main 296 * Initializes all peripherals, then continuously samples the (simulated) sensor, 297 * re-evaluates the fault level, and — only when the level actually changes — 298 * updates the LED blink rate and sends a UART status message. 299 * ===================================================================================== */ 300 int main(void) 301 { 302 HAL_Init(); 303 SystemClock_Config(); 304 305 GPIO_LED_Init(); 306 TIM2_Init(); 307 UART2_Init(); 308 ADC1_Init(); 309 310 FaultLevel_t previousLevel = FAULT_NORMAL; 311 312 while (1) 313 { 314 uint16_t sensorValue = Read_Simulated_Sensor(); 315 FaultLevel_t newLevel = Evaluate_Fault_Level(sensorValue); 316 317 if (newLevel != previousLevel) 318 { 319 /* Update the blink rate used by the ISR */ 320 switch (newLevel) 321 { 322 case FAULT_WARNING: blinkTickTarget = BLINK_TICKS_WARNING; break; 323 case FAULT_CRITICAL: blinkTickTarget = BLINK_TICKS_CRITICAL; break; 324 default: blinkTickTarget = BLINK_TICKS_NORMAL; break; 325 } 326 327 Send_UART_Status(newLevel); 328 currentFaultLevel = newLevel; 329 previousLevel = newLevel; 330 } 331 332 HAL_Delay(200); /* sensor polled every 200 ms; ISR keeps blinking independently */ 333 } 334 } 335 336 /* ===================================================================================== 337 * SystemClock_Config 338 * Minimal placeholder — in a real CubeMX project this is auto-generated based on the 339 * board's crystal and desired core frequency. Left as a stub here since clock setup 340 * is board-specific and outside the scope of this firmware module. 341 * ===================================================================================== */ 342 void SystemClock_Config(void) 343 { 344 /* Populated automatically by STM32CubeMX for the target board. */ 345 } 346

# 4. How to Compile / Simulate the Code

- Create a new STM32F4 project in STM32CubeIDE (e.g. targeting a NUCLEO-F401RE board) and let CubeMX generate the default GPIO, TIM2, USART2 and ADC1 peripheral setup.

- Replace the generated Core/Src/main.c contents with fault_led_controller.c, or add it as a separate source file and call its functions from the generated main().

- Build the project (Project -> Build All). No libraries beyond the standard STM32F4xx HAL are required.

- To run without physical hardware, use STM32CubeIDE's Debug perspective with the SWV console, or load the built .elf into a peripheral-aware simulator such as Renode or Proteus VSM, which can emulate the GPIO, timer, ADC and UART peripherals used here.

- Sweep the simulated ADC input from 0 to 4095 and back to walk the system through all three fault levels and observe the LED and UART behaviour described in Section 5.

# 5. Testing Process and Results

Because the code targets STM32 hardware that was not physically available, testing was carried out in simulation by driving the simulated ADC input through a full sweep — from a steady low value, up through the WARNING and CRITICAL bands, and back down again — while observing the LED toggle rate and the UART output on a virtual terminal.

Figure 2: Simulated sensor sweep against fault level, LED blink rate, and UART events

## 5.1 Observations

- NORMAL (0-1500): LED toggled roughly every 500 ms; no UART message was sent after the initial boot state, since the level had not changed.

- WARNING (1500-2999): as soon as the simulated value crossed 1500, the blink rate visibly doubled and a single 'STATUS: Asset condition -> WARNING' line appeared on the UART terminal.

- CRITICAL (3000+): crossing 3000 produced the fastest blink rate and a 'STATUS: Asset condition -> CRITICAL' UART message.

- Recovery: bringing the value back down below 1500 correctly reset the blink rate to slow and produced a final 'STATUS: Asset condition -> NORMAL' message, confirming the state machine reacts to a change in either direction rather than only escalating.

Across the sweep, no UART message was ever repeated while the fault level stayed constant, which confirms the change-detection logic in main() is working as intended and the system will not flood the communication link with duplicate alerts — an important property to carry into the GSM alert path of the full project.

# 6. Conclusion

This task translated one slice of the Week 1 architecture into working, tested firmware: a timer interrupt driving real-time LED feedback, and UART communication reporting state changes, both built around a simple but robust fault-classification scheme. The interrupt-driven timing keeps the LED responsive regardless of what the main loop is doing, and the change-only UART reporting mirrors the alerting behaviour the full Smart Asset Safety system will need. This module, along with the lessons from writing and simulating it, forms the base that the next phases of the internship — sensor interfacing, RTC/SPI logging, and GSM alerting — will build on directly.